Shift Report

| - Date: 03-02-2025 |  | Shift: Day |
| --- | --- | --- |
| Shift Supervisor | Joey Smith, Ryan Seay |  |
| Cold Side Console | Dane Alexander, Dustin Guidry |  |
| Hot Side Console | James Williamson, Juan Pina |  |
| Furnace Operator | Ramon Zapata, Ronnie Jackson |  |
| Dryer Operator | Corbin Ward, Chaston Tamez |  |
| Compressor Operator | Dana Dickerson |  |
| Fractionation Operator | Kendall Scoville, Justin Chesson |  |
| Offsite Operator | Kenneth Vallier, Will Curet |  |
| Safety Operator | Case Skinner, Les Kolb |  |
| Railcar Loaders |  |  |
| Training | Steven Mire (WWTP) |  |
| Vacation, sick, etc. |  |  |

Weather Event

None.

Safety and Environmental

None.

Process Safety

None.

Unit Limitations / Constraints

Plant down for 2025 TAR.

Operations

Cold Board

- C-1, C-2, & C-3 offline

- Continued working on instrumentation PM list & SIF testing

- Monitored Sphere pressures

- NaOH strengths: { (W)}; { (M)}; { (S)}

- Ethylene production:

Hot Board

| Furnace | Conversions | Feed Rates | Days Online |
| --- | --- | --- | --- |
| F-1A | DOWN | - | - |
| F-1B | DOWN | - | - |
| F-1C | DOWN | - | - |
| F-1D | DOWN | - | - |
| F-1E | DOWN | - | - |
| F-1F | DOWN | - | - |
| F-2 | DOWN | - | - |
| F-201 | DOWN | - | - |

| Dryer | Status | Hours |
| --- | --- | --- |
| R-1A | DOWN | 26.0 @ Shutdown |
| R-1B | DOWN | - |
| R-1C | DOWN | 66.0 @ Shutdown |
| R-2A | Needs regen | Put online 7/14/24 |
| R-2B | Standby | - |
| R-601A | DOWN | 113.0 @ Shutdown |
| R-601B | Standby | - |

- B-1A, B-1B, B-201, & B-501 offline

- C-5A and C-5B Online 02/14/2025.

- Monitored & maintain equipment

Compressors

- Kaizen:

Working on C-2 & C-3

- Brown & Root:

Installed roof over C-1

Installing plates on E-246

Working 12” 610 piping in rack under C-1

Working TM-25 grinding, cutting, fitting, & welding

- Turner

Installed Bypass valve on D-78

- Strong caustic injection strokes set @

- BFW injection to wash water set @

- Water wash to mid caustic section set @

- Current EIP’s

- Bin 1   P-212A

- Bin 2   P-214A

- Bin 4   C-2 lube oil turbine

- Bin 5   C-1 lube oil turbine

- Bin 6   C-2 XV

- Bin 3-10   Empty

Fractionation

- Replaced gaskets on D-25 and closed out

- Installed more valves and spools on SV’s 956A/B

- Welding & fitting continues on SV-941 & SV-6383 lines

- Fitted SV-945/945S sensing line and made field weld

- Debusk setting up to hydro blast 175 tubes on E-43C

- Current EIP's

- Bin 1   T-8B Sight glass

- S. Bin 64   Demethanizer system

- Bin 2-10    Empty

Dryers

- E-601 Capital project is complete & blinds have been removed

- Single source LOTO on XV-10003 (EIP in North LOTO Bin)

- Current EIP's

  - Bin 1   E-14A   Cleaning

  - N. Bin 31   Booster skid

  - Bin 3-10    Empty

Furnaces

- Level in NaOH tote @ IVOL RO is

- Level in NaOH tote @ Veolia RO is

- Mitternight made entry in B-201 to stage tube bundles.

- Mitternight welding supports on B-201 header.

- Turner replacing D-38 ball valve.

- Turner installing new gasket on A-300 manway.

- Turner replacing filters in D-100 A/B.

- Turner insulation reinstalled roof over F-1E penthouse, due to possible bad weather coming.

- Turner pulled check valve on D-77 and valve past inspection, was reinstalled.

- Debusk set up new equipment for D-1 cleaning, work was shut down due to issues with equipment.

- IVOL Instrumentation demo air piping and conduit to B-1B for repairs.

- B&R installed new Decoke valve on F-1A.

- B&R installed new Decoke valve on F-1B.

- B&R installed new Decoke valve on F-1C.

- B&R installed new Decoke valve on F-1D.

- B&R installed new Decoke valve on F-1F.

- B&R installed new Decoke valve on F-2

- B&R installed new Decoke valve on F-201

- B&R made entry into F-1E to remove tubes and make repairs.

- B&R working on installing and welding case drains to P-216A.

- B&R making a weld repair on LV-6003B control loop.

- B&R hydro tested, installed dollar plate, and pulled blinds on E-3C.

- B&R cutting on 3” BFW inlet valve for B-1A and B-1B.

- B&R open manway on V-300.

- B&R removed the South manway on D-1.

- Debusk began vacuuming out D-1 solids, issues with the truck has set them back.

- Single source LOTO on SV-9091

- Single source LOTO on C-5C starter breaker (Temporary air compressor installed)

| Conductivity | 0900 | 1300 | Comments |
| --- | --- | --- | --- |
| D-32A | - | - | LOTO. |
| D-32B | - | - | LOTO |
| D-32C | - | - | LOTO. |
| D-32D | - | - | LOTO. |
| D-32E | - | - | LOTO. |
| D-32F | - | - | LOTO. |
| D-35 | - | - | LOTO. |
| D-201 | - | - | LOTO. |
| B-1A | - | - | LOTO. |
| B-1B | - | - | LOTO. |
| B-201 | - | - | LOTO. |
| B-501 | - | - | DOWN. |

Current EIP's

Furnace North

  - Bin 1   P-216B

  - Bin 2   P-1B   Pull turbine

  - Bin 3   E-304   Tube repairs

  - Bin 5   D-1

  - Bin 6   B-201

  - Bin 4, 7-10   Empty

Furnaces (cont.)

Furnace South

    - Bin 1   F-201   TLE cleaning/Entry

    - Bin 2   F-1A   Entry

    - Bin 3   B-1B   Blinding for C-515B

    - Bin 4   B-1B   Entry

    - Bin 5   F-2   Entry

    - Bin 6   F-1F   Radiant tube repair

    - Bin 10   C-6B Turbine   Coupling Maintenance

    - Bin 7-9   Empty

WWTP/OSBL

OSBL

- Electronic rounds

- Samples

- Vac truck worked on Containments

- Added Chlorine tabs to settling tank as needed

- Cleaned Imhoff baskets, trickling arm, GA-753 suction, & algae

- All cooling tower fans offline

- Reset N2 @ mid-night:

- Current EIP’s

    - Bin 1   Imhoff tank   Bypass valves

    - Bin 2   SV-981 @ P-44s

    - Bin 3   P-41B   Seal repair

    - Bin 4-10   Empty

WWTP

- Electronic rounds

- Samples

- Checked D-308 for condensate

- Cleaned clarifier outer ring as needed

- Pumped down TK-9 sump & cleaned strainers as needed

- Fuel added to compressor

- Lift station offline @ WWTP

- Oil skimmer offline @ CPI

- Current EIP's

    - Bin 1-10   Empty

Shipping

- Current EIP’s

- Bin 4   SV-6357

- Bin 1-3, 5   Empty

Electrical Readings:

- SEL 735A:

- SEL 735B: